JsPrinterDll.dll documentation

a). Wrapping functions：

There are totally 19 functions wrapped in JsPrinterDll.dll file：

Functions to be called on USB port

(1). HANDLE _stdcall OpenUsb( );                      //open USB device

Description: This called function is able to open all series USB devices manufactured by Xprinter. Returned value is the opened USB device handle if succeed, otherwise, it returns INVALID_HANDLE_VALUE.

(2). BOOL _stdcall WriteUsb(HANDLE hUsb,	   //Write data to USB handle

char *SendBuf,  			          //Address pointer of sending buffer

DWORD SendBufSize,					          //Size of sending buffer

LPDWORD lpNumberOfBytesWriten);		//Pointer using to store written

bytes of data . Can’t be NULL.

Description: This called function is able to write data to USB device. Returned value is 1 if succeed. otherwise, it returns 0.

(3). BOOL _stdcall ReadUsb(HANDLE hUsb,   // Read data from USB handle

char  *ReadBuf,  					 	   //Address pointer of read buffer

DWORD  ReadBufSize, 	 				         //Size of read buffer

LPDWORD  lpNumberOfBytesRead);      // Pointer using to store read											    Bytes of data . Can’t be NULL.

Description:This called function is able to read the data sent by USB device. Returned value is 1 if succeed. otherwise, it returns 0.

(4). BOOL CloseUsb(HANDLE hUsb);		  //Close the opened USB handle

Description:This called function is able to close the opened USB handle. Returned value is 1 if succeed. otherwise, it returns 0.

Functions to be called on Lan port

(1). BOOL _stdcall InitNetSev();                // Initialize SOCKET service

Description: This called function is able to initialize SOCKET service. Returned value is 1 if succeed. otherwise, it returns 0. It needs to be called at program initialization.

(2). int _stdcall ConnectNetPort(SOCKET *lpSocket,//Input assigned SOCKET  pointer

SOCKADDR_IN * pPrinterAddr,                    //Printer IP address

timeval *lpTimeout);                               //Input timeout data

Description: This called function is able to connect printer. Returned value is 1 if succeed. Otherwise It returns:

-1：Fail to crate SOCKET.

-2：Fail to set non-blocking mode.

-3：Connection time out.

-4：Select error

(3). int _stdcall WriteToNetPort(SOCKET *lpSocket, //Input assigned SOCKET                pointer

char *SendBuf,                     // Address pointer of sending buffer

DWORD SendBufSize);					         // Size of sending buffer

Description:This called function is able to send data to connected printer via Lan port. Returned value is the sent bytes of data. Otherwise It returns -1.

(4). int _stdcall ReadFromNetPort(SOCKET *lpSocket,      // Input assigned SOCKET pointer

char *RecvBuf,   		              // Address pointer of receiving data

DWORD RecvBufSize);                      // Size of receiving buffer

Description:This called function is able to read the data sent by printer via Lan port. Returned value is the bytes of data if succeed. otherwise, it returns

0：The connection has been terminated.

-1：SOCKET_ERROR

(5). BOOL _stdcall CloseNetPor(SOCKET *lpSocket);      //Close connected   SOCKET

Description: This called function is able to close the connected SOCKET. Returned value is 1 if succeed. otherwise, it returns 0.

(6). BOOL _stdcall CloseNetServ();					 //Close SOCKET service

Description: This called function is able to close the SOCKET service. Returned value is 1 if succeed. otherwise, it returns 0. It needs to be called at the end of program.

Functions to be called on Lpt port

(1). HANDLE _stdcall OpenLptW(LPCWSTR LptName);  //Name of LPT port

Description: This called function is able to open LPT devices handle. Returned value is the opened LPT device handle if succeed, otherwise, it returns INVALID_HANDLE_VALUE.(adapt to Unicode )

(2). HANDLE _stdcall OpenLptA(LPCSTR lpLptName);  // Name of LPT port

Description: This called function is able to open LPT devices handle. Returned value is the opened LPT device handle if succeed, otherwise, it returns INVALID_HANDLE_VALUE.(adapt to Multiple bytes code. For example: GB18030 and GB2312)

(3).BOOL _stdcall WriteLpt(HANDLE hLpt,        // Write data to LPT handle

char *SendBuf,   				      // Address pointer of sending buffer

DWORD SendBufSize,					         // Size of sending buffer

LPDWORD lpBytesWritten);  // Pointer using to store written bytes of data

Description: This called function is able to write data to LPT device. Returned value is 1 if succeed. otherwise, it returns 0.

(4).BOOL _stdcall CloseLpt(HANDLE hLpt);   //Close the opened LPT handle

Description: This called function is able to close the opened LPT device. Returned value is 1 if succeed. otherwise, it returns 0.

Functions to be called on Serial port

(1). HANDLE _stdcall OpenComW(LPCWSTR lpCom,	  //Name of serial port

DWORD BaudRate);                             //Baud rate of printer

Description: This called function is able to open LPT devices handle. Returned value is the opened LPT device handle if succeed, otherwise, it returns INVALID_HANDLE_VALUE.(adapt to transmit Unicode data)

(2). HANDLE _stdcall OpenComA(LPCSTR lpCom,      //Name of serial port

DWORD BaudRate);                             // Baud rate of printer

Description: This called function is able to open Serial devices handle. Returned value is the opened Serial device handle if succeed, otherwise, it returns INVALID_HANDLE_VALUE.(adapt to transmit Multiple bytes code data. For example: GB18030 and GB2312)

(3). BOOL _stdcall ReadCom(HANDLE hCom,    // Read data from COM device handle

char * ReadBuf,                       // Address pointer of read buffer

DWORD ReadBufSize,  		                    //Size of read buffer

LPDWORD lpNumberOfBytesRead);        // Pointer using to store read													            bytes of data

Description: This called function is able to read the data sent by COM device. Returned value is 1 if succeed. otherwise, it returns 0.

(4). BOOL _stdcall WriteCom(HANDLE hCom,	   //Write data to COM device

handle

char *SendBuf,			             // Address pointer of sending buffer

DWORD SendBufSize,		                    // Size of sending buffer

LPDWORD lpBytesWritten);  // Pointer using to store written bytes of data

Description: This called function is able to write data to COM device. Returned value is 1 if succeed. otherwise, it returns 0.

(5). BOOL _stdcall CloseCom(HANDLE hCom);     //Close the opened Com

device handle

Description: This called function is able to close the opened COM handle. Returned value is 1 if succeed. otherwise, it returns 0.

b). How to call the functions in JsPrinterDll.dll file：

Standard calling convention: stdcall

(1).Implicit linking

(2).Display linking

c). Cautions：

Don’t call OpenUsb() and CloseUsb() function too frequently. If OpenUsb() is called less than 1s interval after CloseUsb() called. There may be error while writing or reading data. More so, it may cause system crashed.